JOAN YOLANDE
Victorian River Launch

The original launch “Victoria” was built in 1912 by Simpson, Strickland & Co, of Dartford, Devon as a private river launch. Motive power was likely to have been a compound, surface condensing, steam engine with a coal fired boiler. Victoria is believed to be preserved and is now located somewhere in the U.S.A.

The model was purchased from Cheddar Models (no longer trading) and built over two year period from the kit “Victoria” manufactured by Klaus Krick Modeltechnik. The hull is an A.B.S. moulding with a length of 1077 mm (43.5”) and a beam of 253 mm (10.5”) making the scale 1:10.
Various strip and block woods were used in the construction such as Mahogany, Tanganjica, Pine, Obeche, Lime and Plywood for the foundation structures, along with several Brass fittings and ready made rope fenders.

The cabin awning, plus a stepladder and a seat in the engine compartment were not constructed, due to aesthetics and lack of space

However, additional features and items have been used such as Brass fittings from Marten Howes and Baylis flags from Hand Made Flags, scale rope from David H Alderton Models, a steam cock from Bruce Engineering and Mahogany strip from Mantua Model UK & Mobile Marine Models.
Motive power is supplied by a Cheddar Models Gemini this is a two cylinder, in line, slide valve steam engine, which has a Bore of 9.7mm (3/8”) and a Stroke of 11.0 mm (7/16”). Forward & reverse is controlled by a Maudslay style gear operated via a remote control servo. Drive is direct through a 360mm (12”) prop shaft to a 70 mm (2.75”) four blade propeller. Two other servo’s control the steam valve throttle and rudder. 

The vertical boiler with ceramic burner is gas fired via a refillable gas tank containing a 70%/30% mixture of Butane/Propane. Providing superheated steam with working pressures of between 40 psi (3 Bars) and up to a maximum of 100 psi (6 Bars). Running time is between 25 to 35 minutes approximately being dependant on boiler capacity (0.6Lt), gas temperature and pressure which can be influenced by the chilling effect as gas is drawn off. Plus steam valve throttle control.
The name Joan Yollande is in memory of a sweet and gentle lady, Joan Yolande Aderton (1914 -1999) who’s generosity lead to the purchase of the Gemini steam engine. 
God Bless Her.
Bruce Engineering : www.pollymodelengineering.co.uk
Martin Howes & Baylis : www.Model-Steam-Boats.co.uk
Mobile Marine Models : www.mobilemarinemodels.com
David H Alderton (Master Shipwright) , 1 Old School Cottages, Creech St Michael

 01823443754                                                                                             Taunton TA3 5QQ    
Hand Made Flags – Mike Doane   64 Copse Avenue , West Wickham, Kent.

 02084021607                                                                      E-mail  handmadeflags @aol.com

Mantua Models UK   Windsor, Berkshire 
(attend International Model Boat Show , Warwickshire in November)
